REPUBLIC OF ARMENIA

LOCAL ECONOMY AND INFRASTRUCTURE DEVELOPMENT PROJECT

ENVIRONMENTAL AND SOCIAL MANAGEMENT PLAN

“Reconstruction of tourism infrastructure and roads of Gyumri city”

TUR-13.3 “Restoration of the Park Adjacent to the St. Amenaprkich Church and Right and Left Sections of the Adjacent Abovyan Street”

March 2023

PART A: General Project and Site Information

| INSTITUTIONAL & ADMINISTRATIVE |  |  |  |
| --- | --- | --- | --- |
| Project title | LOCAL ECONOMY AND INFRASTRUCTURE DEVELOPMENT |  |  |
| Subproject number and title | TUR-13, Reconstruction of Tourism Infrastructure and Roads of Gyumri City Component 3. TUR-13.3. "Restoration of the Park Adjacent to the St. Amenaprkich Church and Right and Left Sections of the Adjacent Abovyan Street" |  |  |
| Municipality, community | Gyumri City, Shirak marz |  |  |
| Scope of site-specific activity | The main goal of the subproject is to create a well-organized, modern tourist infrastructure in harmony with the general style of Gyumri, to improve important parks of the city, public spaces of the historical center of the city, road network and engineering infrastructure. Ensure uninterrupted operation of city transport and tourist routes and accessibility of historical and cultural monuments and tourist facilities in the area. The subproject includes 6 components, of which this ESMP covers Component 3. “Restoration of the Park Adjacent to the St. Amenaprkich Church and Right and Left Sections of the Adjacent Abovyan Street”. The component includes the following activities: Right and left sections of Abovyan street Abovyan Street stretches from the intersection of Vahan Cheraz Street to the intersection with Teryan Street, total- 0.62 km. The works will include restoration of the roadway (stone and asphalt paving) and sidewalks of Abovyan Street passing on both sides of the park, restoration of the rainwater removal system, repair of the sewer line, repair/reinstallation of electricity supply, communication, gas supply lines, dismantling of non-functioning lines, installation of external lighting, adaptation of the area for the people with limited mobility, furnishing (installation of benches, trash bins), repair of thresholds, adaptation of the territory for population groups with limited mobility, such as provision of ramps, and addressing special measures for people with visual impairments, etc. Park Area next to St. Amenaprkich Church: repair of the retaining wall of the park, replacement of paving, repair and partial reconstruction of the stairs leading to the church and the four sides of the fountain, reconstruction of the non-functioning fountain, installation of necessary equipment, construction/reconstruction of the water supply and drainage systems of the fountain, modernization of the electrical supply pillars installed in the park, installation of external lighting, adaptation of the area for the people with limited mobility (including ramps), improvement, reconstruction and landscaping of lawns, planning of the playground, furnishing of the park (installation of benches, trash bins, tourist signboards). The restoration activities will cover entire area of the park, which is 1.6 hectares. |  |  |
| Institutional arrangements (WB) | Task Team Leader: Tafadzwa Dube | Safeguards Specialists: Oksana Rakovych (Environment) Lusine Gevorgyan (Environment) David Jijelava (Social) |  |
| Implementation arrangements (RoA) | Implementing entity: ATDF Asya Osipova (Environment) Knarik Grigoryan (Social) | Works supervisor: Bagarani Bariq CJSC | Works contractor: Merdzmoskovyan OJSC |
| SITE DESCRIPTION |  |  |  |
| Name of institution whose premises are to be rehabilitated | Gyumri City |  |  |
| Address and site location | Shirak marz, Gyumri community, the park adjacent to the St. Amenaprkich Church and right and left sections of the adjacent Abovyan Street |  |  |
| Who owns the land? Who uses the land (formal/informal)? | The sub-project implementation does not require acquisition of private land, pasture, water, public services, or other resources. The road and the sidewalks to be rehabilitated are public property. No businesses or kiosks are located on Abovyan street. Two kiosks are located at the entrance of the park to be rehabilitated, from which one is operating. The rehabilitation works will not impede the operation of the kiosk. A safe access will be provided for the kiosk during the rehabilitation works. Rehabilitation of exterior staircases of 3 basements of apartment building (Abovyan street, buildings num. 252 and num. 254 ), and entrances of 5 houses located on Abovyan street is planned under the Component. The owners of the houses (5) and users (2) of the basements are informed about design solutions. Consent forms on the implementation of the rehabilitation works have been signed by all the owners of the houses and users of the basements in cooperation with Gyumri Administrative office (an example is attached, see Attachment 4). |  |  |
| Description of physical and natural environment around the site | Abovyan Street is located in the center of Gyumri and within the “Kumayri” historic district, also known as Kumayri Historical and Cultural Museum-Reserve, which is under the state protection. Between the left and right parts of the street there is a St. Amenaprkich Church and а city park (1.6 hectares). The territory of the park is not well maintained, in some places there are accumulations of household waste. There are many large trees in the park, including acacia, maple, wild apple, pear, plum and other wild relatives of cultivated plants. No tree will be impacted during the implementation of the project. There is a non-functioning fountain on the territory of the park, which will be reconstructed, the necessary equipment for the drainage of the fountain will be installed. The climate is continental (long cold winters, moderately hot summers) dry. In winter, there are severe frosts - down to -41 ° C. In summer, it is relatively hot, up to + 36 ° C. During the year, there is little precipitation - 500 mm on average. The restoration of this central part of the city of Gyumri will improve the quality and reliability of public infrastructure; will support the development of tourism, especially the attendance of the tourist flow to the St. Amenaprkich Church and park . After the restoration of the engineering infrastructure of the sidewalks, the installation of new lighting fixtures, rubbish bins and benches, Abovyan Street will acquire a new color and become more lively than today. Architectural solutions, landscaping elements will be selected in harmony with the historical and architectural environment, the building materials used will be durable, easy to clean, all materials used will be resistant to temperature fluctuations. The available information on the environment is sufficient to describe the baseline conditions in the subproject areas and the magnitude of the impact on the ecosystem from the proposed activity. It is expected that the negative impact of the subproject on the environment will be small, temporary and will not extend beyond the subproject area. |  |  |
| LEGISLATION |  |  |  |
| National & local legislation & permits that apply to sub-project activity | The following Armenian legislation defines a legal framework applicable to project activities: - Law on Atmospheric Air Protection of RoA (1994) The purpose of the Law on Atmospheric Air Protection is to define the main principles directed to the provision of purity of atmospheric air and improvement of air quality, prevention and mitigation of the chemical, physical, biological and other impacts on air quality and regulation of public relation. This Law also regulates the emission licenses and provides maximum allowed loads/concentrations for atmospheric air pollution, etc. According to this law, the contractor shall undertake construction activities as well as transportation and temporary storage of waste the way to minimize dust and other emissions. - Law on Waste of RoA (2004) The law provides the legal and economic basis for the collection, transportation, disposal, treatment, re-use of wastes as well as prevention of negative impacts of waste on natural resources, human life and health. The law defines the roles and responsibilities of the state authorized bodies as well as of waste generator organizations in waste management operations. According to this law, the waste generated from construction activities should be recycled as appropriate or disposed of in designated locations. Disposal of the construction waste and excess material in selected locations must be approved by the local municipality in writing. - Law on Environmental Impact Assessment and Expertise of RoA (2014) The law defines the type of activities that are subject to environmental impact assessment and environmental expertise. - Law of the RoA on “Preservation and Use of Immovable Monuments of History and Culture and Historic Environment” According to this laws, the proposed works are subject to environmental impact assessment and expert examination, as Abovyan street to be rehabilitated under the Component is located within the “Kumayri” historic district, , as well as the “Amenaprkich” church is included in the state list of cultural monuments of RoA. The positive conclusion of the State Environmental Impact Expertise is attached (Attachment 3). - Law on Urban Development of RoA (1998) According to this law, planned works require obtaining the construction permit. |  |  |
| PUBLIC CONSULTATION |  |  |  |
| When / where the public consultation process will take /took place | Five public consultation meetings have been conducted under the sub-project covering the Component design solutions and mitigation measures of expected environmental and social impacts. The public consultation meetings were broadcasted on the official web page and YouTube channel of Gyumri municipality. Four public consultation meetings were conducted under state EIA and expertise procedure with the participation of residents of the streets to be rehabilitated under the Components, Administrative office of Gyumri and director of "Kumayri" Historical and Cultural Reserve-Museum public non-profit organization. One more public consultation meeting was conducted (on September 8, 2022) with the participation of the owners of the houses whose staircases and entrances will be rehabilitated under the sub project. The design solutions and the content of the consents to be signed were presented during the meeting. The draft ESMP was disclosed and discussed at the public consultation meeting held in Gyumri city on April 14, 2023. Draft ESMP was disclosed in Armenian and English languages on the ATDF website (atdf.am) and in the Armenian language on the beneficiary's website (gyumricity.am) and social media account. Brief information on the planned works and contact information for addressing questions and grievances will be placed on the information board of the Gyumri municipality, at the worksite, and/or in its immediate surroundings. |  |  |
| ATTACHMENTS |  |  |  |
| Attachment 1: Site map/photo Attachment 2: Copy of permission for construction waste disposal Attachment 3: The positive conclusion of the State Environmental Impact Expertise Attachment 4: Consent of the owner of the house with staircase to be rehabilitated under the Component Attachment 5: Minutes of the public consultation on the draft ESMP Attachment 6: Construction permit |  |  |  |

PART B: safeguards information

| ENVIRONMENTAL /SOCIAL SCREENING |  |  |  |
| --- | --- | --- | --- |
| Will the site activity include/involve any of the following? | Activity/Issue | Status | Triggered Actions |
|  | - Road rehabilitation | [x] Yes [ ] No | If “Yes”, See Section A below |
|  | - New construction | [ ] Yes [x] No | If “Yes”, See Section A below |
|  | - Individual wastewater treatment system | [ ] Yes [x] No | If “Yes”, See Section B below |
|  | - Historic building(s) and districts | [x] Yes [] No | If “Yes”, See Section C below |
|  | - Unexpected need for land take | [ ] Yes [x] No | If “Yes”, See Section D below |
|  | - Hazardous or toxic materials | [ ] Yes [x] No | If “Yes”, See Section E below |
|  | - Impacts on forests and/or protected areas | [ ] Yes [x] No | If “Yes”, See Section F below |
|  | - Traffic and pedestrian safety | [x] Yes [ ] No | If “Yes”, See Section H below |
|  | - Social risk management | [x] Yes [ ] No | If “Yes”, See Section I below |

PART C: Mitigation measures

| ACTIVITY | PARAMETER | MITIGATION MEASURES |
| --- | --- | --- |
| 0. General Conditions | Notification and Worker Safety | - The local construction and environment inspectorates and communities are notified of upcoming activities - The public is notified of the works through appropriate notification in the media and/or at publicly accessible sites (including the site of the works) - All legally required permits are acquired for construction and/or rehabilitation - The Contractor formally agrees that all work will be carried out in a safe and disciplined manner designed to minimize impacts on neighboring residents and environment. - Workers’ personal protection equipment complies with international good practice (always hardhats, as needed masks and safety glasses, harnesses and safety boots) - Appropriate signposting of the sites informs workers of key rules and regulations to follow. |
| A. General Rehabilitation and /or construction activities | Air Quality | - During earth works, dust is suppressed by ongoing water spraying and/or installing dust screen enclosures at site. - The surrounding environment (sidewalks, roads) is kept free of debris to minimize dust. - There is no open burning of construction / waste material at the site. - There is no excessive idling of construction vehicles at sites. |
|  | Noise | - Construction noise is limited to restricted times agreed to in the permit. - During operations, the engine covers of generators, air compressors and other powered mechanical equipment are closed, and equipment placed as far away from residential areas as possible. |
|  | Water Quality | - Appropriate erosion and sediment control measures are applied such as e.g. hay bales and / or silt fences to prevent sediment from moving off site and causing excessive turbidity in nearby streams and rivers. - Rainwater is removed from the construction site using a rainwater drainage system. |
|  | Waste Management | - Waste collection and disposal pathways and sites are identified for all major waste types expected from demolition and construction activities. - Mineral construction and demolition wastes are separated from general refuse, organic, liquid and chemical wastes by on-site sorting and stored in appropriate containers. - Construction waste is being collected and disposed properly by licensed collectors. - Whenever feasible, appropriate and viable materials (except asbestos) are reused or recycled. |
|  | Earth Works | - Excess material and topsoil generated from earth works will be stored separately and used for backfilling and site reinstatement - Topsoil layer not thicker than 30 m will be stripped separately and temporary piled on-site - Stockpile height will not exceed 3.0 m, and the slope angle will not exceed 34o - Temporary stockpile will be compacted and covered with a tarp to prevent erosion caused by rain runoff - Topsoil will be re-used for backfilling to facilitate re-cultivation of the land plot - Excess amount of subsoil will be disposed at the agreed-upon locations. |
| C. Historic building(s) and districts | Cultural Heritage | - If the building is a designated historic structure, very close to such a structure, or located in a designated historic district, notification shall be made and approvals/permits be obtained from local authorities and all construction activities planned and carried out in line with local and national legislation. - It shall be ensured that provisions are put in place so that artifacts or other possible “chance finds” encountered in excavation or construction are noted and registered, responsible officials contacted, and works activities delayed or modified to account for such finds. |
| H. Traffic and pedestrian safety | Direct or indirect hazards to public traffic and pedestrians by construction activities | - In compliance with national regulations, the construction site is properly secured, and construction related traffic regulated. This includes but is not limited to - Signposting, warning signs, barriers and traffic diversions: site will be clearly visible, and the public warned of all potential hazards. - Traffic management system and staff training, especially for site access and near-site heavy traffic. Provision of safe passages and crossings for pedestrians where construction traffic interferes. - Adjustment of working hours to local traffic patterns, e.g. avoiding major transport activities during rush hours or times of livestock movement. - Active traffic management by trained and visible staff at the site, if required for safe and convenient passage for the public. - Ensuring safe and continuous access to office facilities, shops and residences during renovation activities, if the buildings stay open for the public. |
| I. Social risk management | Public relationship management | - Before the start of construction works ensure that the construction site is clean of privately owned kiosks located on the public land, according to the design solutions. - Inform ATDF if usage of private land is required during the construction activities and/or in case construction activities impact private property or business. - Assign local liaison person in charge of communication with and receiving requests/ complaints from local population. - Consult local communities to identify and proactively manage potential conflicts between an external workforce and local people. - Raise local community awareness about sexually transmitted disease risks associated with the presence of an external workforce and includes local communities in awareness activities. - Inform population about construction and work schedules, interruption of the services, traffic detour routes and provisional bus routes, blasting and demolition, as appropriate. - Limit construction activities at night. When necessary, carefully schedule night-time works and inform affected community so they can take necessary measures. - At least five days in advance of any service interruption (including water, electricity, telephone, bus routes), advice affected community through postings at the project site, at bus stops, and in affected homes/businesses. |
|  | Labor management | - To the extent possible, locate work camps away from local communities. - Undertake sitting and operation of worker camps in consultation with neighboring communities. - Recruit unskilled or semi-skilled workers from local communities to the extent possible. Where and when feasible, provide worker skills training to enhance participation of local people. - Provide adequate lavatory facilities (toilets and washing areas) in the work site with adequate supplies of hot and cold running water, soap, and hand drying devices. Establish temporary septic tanks for any residential labor camp and without causing pollution of nearby watercourses. - Raise awareness of workers on overall relationship management with local population, establish the code of conduct in line with international practice and strictly enforce them, including the dismissal of workers and financial penalties of adequate scale. |
|  | Grievance Management | - GRM is established and advertised at the local level. The Contractor is informed of his responsibility to register and communicate to the PIU received verbal and non-verbal grievances. - GRM contact information, including local focal points, has been posted at the construction site and the local community’s administrative office. - The Contractors' local liaison person receives and registers grievances from the population in the GRM log and communicates the grievances to the PIU regularly. |

PART D: Monitoring Plan

| Activity | What (Is the parameter to be monitored?) | Where (Is the parameter to be monitored?) | How (Is the parameter to be monitored?) |  |  | When (Define the frequency / or continuous?) | Why (Is the parameter being monitored?) |  |  | Who (Is responsible for monitoring?) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CONSTRUCTION PHASE |  |  |  |  |  |  |  |  |  |  |
| Supply of construction materials | Purchase of construction materials from the registered suppliers. | In supplier’s office or warehouse | Verification of labels of the materials and/or certificates if any |  |  | During conclusion of supply contracts | Ensure reliability of construction materials and their safety for human health |  |  | ATDF Technical supervision company |
| Transportation of construction materials and waste Movement of construction vehicles and machinery | - Technical condition of construction vehicles and machinery; - Confinement and protection of truckloads with lining; - Movement of construction vehicles and machinery according to predefined traffic routes and within working hours: 9AM-6PM. | -Construction site -Routes of transportation of construction materials and wastes | Inspection of pre-defined movement routes, entrances and exits of construction sites |  |  | Unannounced monthly inspection during work hours and beyond | -Limit pollution of soil and air from emissions; -Limit nuisance to local communities; -Minimize traffic disruption, ensure pedestrian safety and prevent accidents. |  |  | ATDF Technical supervision company Road Police |
| Maintenance of construction equipment | - Washing of cars and construction equipment outside the construction site or on maximum distance from natural streams; - Refueling or lubrication of construction equipment outside the construction site or at the predetermined confined area. | Construction site | Inspection of activities |  |  | Selective inspections during work hours | -Avoid pollution of water and soil with oil products due to operation of equipment; -Timely localize and decrease expected damage in case of fire. |  |  | ATDF Technical supervision company |
| Generation of construction waste | - Temporary storage of waste at the pre-defined areas of construction site; - Timely disposal of waste to the formally designated locations. | Construction site Waste disposal site | Inspection of site |  |  | Unannounced monthly inspections during entire construction phase | -Prevent pollution of soil, surface water and ground water; - Avoid accidents at the construction site due to scattered fragments of construction materials and debris; - Retain esthetic appearance of the construction site and its surroundings. |  |  | ATDF Technical supervision company |
| Generation of construction runoff | - Arrangement and regular cleaning of drainage system for rainwater and runoff collection and drain. | Construction site and construction base | Visual observation |  |  | Total period of works | - Reduce pollution of surface and groundwater, soil. |  |  | ATDF Technical supervision company |
| Workers’ health and safety | - Provision of uniforms and safety gears to workers and enforcement of their use by the constructor; - Provision of work-site safety instruction to contractor’s personnel and instructions proper recording; - Strict compliance with the national regulations for construction works; - Presence of fire extinguishing and medical aid kits at work site. | Construction site | Inspection of activities Inspection of records |  |  | Periodically during construction and upon its completion | -Exclude probability of accidents; -Protect workers’ health. |  |  | ATDF Technical supervision company |
| Works near private land plots and other property | - Provisions of nuisance activities within working hours from 9AM-6PM and in accordance with allowed norms of noise and vibrations; - Installation of fences; - No parking of construction vehicles and machinery outside the construction site; - No blocking of pedestrian and vehicle movement around the construction site due to stockpiling/ dumping of construction materials/waste; - No trespassing of private land plots and/or other property around the construction site by contractor’s personnel. | Works near privately-owned land, buildings and other assets | Inspection of activities |  |  | Entire period of construction | -Reduce risks for the neighboring communities; -Reduce the probability of damages to private property -Exclude temporary or permanent violation of ownership and/or user rights on the private property |  |  | ATDF Technical supervision company |
| Tree preservation | - Inventory of existing trees on sub-project site; - Existing trees shall be marked and cordoned off with fencing, their root system protected, and any damage to the trees avoided; - Permit will be obtained from the Gyumri municipality before removal of any trees, bushes and stumps. | Construction site | Visual inspection |  |  | Entire period of construction | Avoid damage to existing vegetation. |  |  | ATDF Technical supervision company Gyumri municipality |
| Construction site re-cultivation | Final cleaning of the site and permanent access roads | Construction site and access roads | Inspection of activities |  |  | Final period of construction | Reduce loss of aesthetical value of the landscape due to construction activities. |  |  | ATDF Technical supervision company |
| Solid waste management | Trash bins placement and arrangement on periodical transportation of waste | Construction site | Inspection |  |  | Total period of works | - Prevent pollution of soil, surface water and ground water; - Retain esthetic appearance of the sites area and its surroundings. |  |  | ATDF Technical supervision company Gyumri municipality |
| Dust generation | Dust suppression measures at worksites | Construction site and access roads | Visual inspection |  |  | Entire period of construction | Prevent off-site dust spreading and pollution; Reduce health risks to the workers and neighboring communities related to dust exposure. |  |  | ATDF Technical supervision company |
| Handling of chance finds | - Stoppage of works promptly upon encountering of chance finds; - Formal communication with the Ministry of Education, Science, Culture and Sports (MoESCS) of RoA on chance finds; - Timely arrangements made for the conduct of required excavation and site conservation. | Construction site | -Inspection of chance find sites; - Inspection of document (s). |  |  | Entire period of construction | Ensure cultural heritage is not damaged by the construction works. |  |  | ATDF Technical supervision company MoESCS |
| Works near the cultural heritage buildings or districts | - Planning and implementation of the construction works near the cultural heritage buildings in line with national legislation; - Avoid incidentally damaging of the cultural heritage buildings. | Construction site | -Inspection of documentation - Inspection of activities |  |  | Entire period of construction | Prevent any damage and loss of cultural heritage․ |  |  | ATDF Technical Supervision company MoESCS "Kumayri" Historical and Cultural Reserve-Museum public non-profit organization |
| Earthworks | - Topsoil removal and temporary stockpiling for cultivation of the land; - Temporary storage of excavated soil at determined places; - Backfilling the excavated ground using subsoil and topsoil from the designated locations and disposal of the excess mass to the places as stated in the reference letter provided by Gyumri municipality. | Construction site | Inspection of activities |  |  | During earth works | - Reduce loss of soil and vegetation due to ground piling and minimization of pollution of surface water reservoirs with particles; - Limit pollution with contaminated soil of surface and ground waters. |  |  | ATDF Technical supervision company |
| Operation of Grievance Redress Mechanism | - Public right to complaints or questions about the project. - Appeal to the head of local government (community authorities) - The appointment by the authorities of the Coordinator for the consideration of complaints (receiving complaints, reviewing and explaining, registering complaints in the logbook and trying to resolve them on the spot. - Appeal to ATDF by hot-line (010 24 01 59), postal letter (Yerevan 0037, K.Ulnetsi 31), official web page (http://atdf.am/hy/Home/ContactUs), e-mail (support@atdf.am). | Construction site | Inspection of activities |  |  | Entire period of construction | -Monitor compliance with project aspects; -Timely resolve questions and complaints in the field. |  |  | ATDF Technical supervision company |
| Traffic and pedestrian safety | - Signposting, warning signs, barriers and traffic diversions: construction site visible and the public warned of all potential hazards; - Traffic management by trained and visible staff at the site for safe passage for the public; - Provision of safe passages and crossings for pedestrians where construction traffic interferes; - Adjustment of working hours to local traffic patterns to avoid major transport activities during rush hours; - Safe and continuous access to nearby office facilities, shops and residences during construction works. | Construction site | Inspection of activities |  |  | During the works | - Minimize direct or indirect hazards to public traffic and pedestrians by construction activities; - Minimize traffic disruption. |  |  | ATDF Technical supervision company Road Police |
| OPERATION PHASE |  |  |  |  |  |  |  |  |  |  |
| Operation and maintenance of lighting system | Regular servicing of the lighting system is being undertaken. | Construction site |  | Periodic inspection of lighting network | Total period of operation |  |  | Maintain road lighting system in safe and operable condition. | Gyumri municipality |  |
| Maintenance and protection of site | Non-authorized construction and land use along the reconstructed roads. | Construction site |  | Visual inspection | Total period of operation |  |  | Prevent the loss of aesthetical and functional value of the roads. | Gyumri municipality |  |
| Solid waste management | Trash bins placement and arrangement on periodical transportation of waste. | Construction site |  | Inspection of activities | Total period of operation |  |  | Maintain safe sanitary condition and prevent environmental pollution. | Gyumri municipality |  |
| Operation and maintenance of utility networks (water supply, sewage, power supply) | Regular servicing of the utility networks being undertaken. | Construction site |  | Regular technical inspections | Total period of operation |  |  | - Maintainutility networks in safe and operable condition; - Prevent traffic disruption and inconvenience to pedestrians and drivers. | Gyumri municipality |  |

Attachment 1: Site map/photo

| Photo 1: The fountain area | Photo 2: Stairs near the monument | Photo 3: Left side of Abovyan street | Photo 4: Left side of Abovyan street |
| --- | --- | --- | --- |
| Photo 5: Left side of Abovyan street | Photo 6: Right side of Abovyan Street | Photo 7: Kiosks located at the entrance of the park, on the left from the Church |  |

Attachment 2: Permission for construction waste disposal

English summary of the above document:

REPUBLIC OF ARMENIA, SHIRAK MARZ, GYUMRI CITY

22.02.2022

REFERENCE N 03/2572-2022

In connection with your letter, we suggest the following locations for the transportation of the  construction waste generated during the construction works and for the required amount of topsoil and  subsoil for the implementation of the restoration works of the streets mentioned in your letter,:

| N | Street name | Construction waste | Reserveof subsoil | Topsoil |
| --- | --- | --- | --- | --- |
| 1 | Khanjyan, Chaykovsky st/km | 7.3 | 12.9 | 7.9 |
| 2 | Haghtanak avenue, km | 9.4 | 10.2 | 10.1 |
| 3 | The right and left parts of the park adjacent to St. Amenaprkich Church and the adjacent Abovyan Palace, km | 7.9 | 10.3 | 8.5 |
| 4 | Ajemyan, Hamesoglyan and Jivan st/km | 8.1 | 10.4 | 8.9 |
| 5 | Teryan and Nalbandyan st/km | 8.3 | 10.5 | 9.0 |

Leader                        Sealed/Signed                        D.Arushanyan

Attachment 3: The positive conclusion of State Environmental Impact Expertise

Summary of the above document

EE No 0133-22

Expertise Initiator:  "Haldi Consalt" LLC, Gegharquniq  marz, Noratus village.

Activity: Environmental Impact Assessment Report for ”Restoration of Khanjyan and Tchaikovski Streets, Haghtanak Avenue, the Park Adjacent to St Savior Church and Right and Left sides of Abovyan Street, Achemyan, Hanesoghlyan and Jivani Streets, Teryan and Nalbandyan Streets of Gyumri City”   subproject.

The expertise requirements:

- During the implementation of the construction works, it is necessary to monitor the sanitary norms of the area, to implement the environmental measures specified in the assessment report, and in case of non-satisfaction, to plan additional measures.

- It is necessary to carry out the planned construction works during the specified hours of the day, minimizing as much as possible the noise and the maximum load on the roads.

- Carry out concrete transportation in the manner specified in the report by excluding concrete production at the construction site or adjacent areas.

- During the construction works, ensure the requirements on the environment, as well as human health, safety and technical safety standards of the RoA.

- In accordance with the request of the RoA Government's decision N 438 dated April 20, 2002, in case of discovery of historical-cultural or archaeological layers during the implementation of activities, it is necessary to stop the works, informing the authorized body. Resume the works only after the permission of the authorized body.

EXPERTISE CONCLUSION

A positive conclusion is given to the Environmental Impact Assessment report of the Reconstruction of Tourism Infrastructure and Roads of Gyumri sub-project, submitted by the "Haldi Consult" LLC, with the obligatory fulfillment of expert requirements.

"Environmental Impact Assessment and Expertise Center" SNCO, senior specialist Armen Vardanyan.

Attachment 4: Consent of the owner of the house with staircase to be rehabilitated under the Component

Non-official translation of the document:

To the owner of building N266 of the

Abovyan Street of Gyumri community

Argina Asatryan

Agreement

Abovyan street is under design in the frameworks of “Reconstruction of Tourism Infrastructure and roads of Gyumri city” sub project.  We are asking for your consent to repair the staircase leading to your house. The repair works will be included in the construction works to be conducted on Abovyan Street. The construction works will be financed by Armenian Territorial Development Fund.

I agree for which I sign _______signature

Attachment 5: Minutes of the public consultation on the draft ESMP

Public consultation was held in the Gyumri community municipality office on March 24, 2023, within the framework of the "Reconstruction of tourism infrastructures and roads of Gyumri city" subproject of the Local Economy and Infrastructure Development Program.

The announcement of the public consultation was posted on the ATDF and Gyumri municipality websites on March 15. The draft ESMPs were posted on the web pages of ATDF and Gyumri community.

Overall, 20 participants were present at the public consultation, of which 6 were women. The representative from ATDF (social specialist), "Haldi Consult" design organization, Gyumri Administrative office, civil society and media, and members of the City council, were present at the discussion. The owners of the kiosks located on the sites were notified of the date of the public consultation by the Administrative office of Gyumri. A representative of the traders was present at the meeting.

The public consultation was broadcasted on the official YouTube channel of Gyumri municipality and was viewed more than 600 times during the following five days. Within the five days following the public consultation, ATDF has not received any additional observations or comments.

During the public consultation, the three ESMPs covering five Components of the subproject were presented, which included the restoration works planned for the park of the St. Amenaprkich Church and the right and left sides of Abovyan Street, Khanjian and Chaikovsky Streets, “Haghtanak” Avenue, Teryan and Nalbandyan, Achemyan, Jivan and Hanesoghlian streets.

The deputy head of Gyumri community opened the meeting by presenting the purpose of the public consultation and the aim of the subproject.

The Social Specialist of ATDF presented the expected environmental and social impacts and mitigation measures within the framework of the above-mentioned components of the sub-project, including air, water quality, noise, traffic and waste management, ensuring the provision of safe access to kiosks located in the sub project sites, as well as occupational health and safety measures.

It was mentioned that no impact on the private lands is anticipated under the Components.

The GRM procedures were presented. The focal point of the subproject was presented and additionally informed about his responsibilities.

Questions and Answers:

- Question- Is it planned to plant new trees and landscaping the area adjacent to the Amenaprkich Church? Is an irrigation system planned?

Answer- Planting of new trees is not envisaged under the subproject, and tree cutting is not planned either. The area does not have an irrigation system, green areas and trees will be watered using watering machines.

- Question - There is a problem of water accumulation in the basements of the buildings on Abovyan street, is there any measures planned in this regard within the framework of the subproject?

Answer - The sub project provides for a proper drainage system for that road section. As a result of improperly organized drainage of the road water accumulated fills the basement of the building located on Abovyan Street. The solutions provided under the new design, including the replacement of the manholes and the underground pipeline will provide proper drainage of the road and prevent the flow of water to the basement of the building.

- Question- What solutions have been given regarding the temporary kiosks located along the roads?

Answer: In accordance with the scope of the Resettlement Policy Framework of the Project, a screening of all possible negative social impacts was carried out. This sub-project does not suppose the relocation of the kiosks. At the preliminary stage of the evaluation of the subproject, the Gyumri municipality submitted a statement to ATDF, according to which if any unexpected resettlement come up, compensation would be carried out by the municipality. A particularly large number of kiosks are located along Haghtanak avenue, but no relocation is planned under the Component. All works on site will be organized in a way that the works do not harm any businesses; and a safe access will be provided to all kiosks.

- Question: When will construction works begin? How long will the works last?

Answer: It is planned to start the construction works under all components during the next quarter. Construction should be completed by the end of 2024.

- Question: Is the culvert passing through the middle section of Haghtanak Avenue included in the project?

Answer: The drainage system of the two road sections of Haghtanak avenue will be completely reconstructed including the drainage system, which will be carried out through an underground system., but the intermediate park section Victory park under which the separate culvert passes is not included in the sub project, only the fences of the park will be repaired. The new underground drainage system of the road to be constructed under the sub project and the existing culvert passing under the park are not connected. The park section of the avenue was not included in the sub project.

Attachment 6: Construction Permit

Summary of the above attached document:

Republic of Armenia

Shirak Region, Gyumri Community

Construction Permit N148/23, 09.08.2023

This is given for the «Restoration of the Park Adjacent to the St. Amenaprkich Church and Right and Left Sections of the Adjacent Abovyan Street»

The design documents are

1.	Developed by “Haldi Consult” LLC, License 16919

2.	The reconstruction will be implemented within 12 month.

A.Papikyan                    Sealed/Signed

Toxic / hazardous material includes but is not limited to asbestos, toxic paints, noxious solvents, removal of lead paint, etc.